SECTION 27 05 29.13

HANGERS AND SUPPORTS FOR A-V EQUIPMENT

GENERAL

SUMMARY

Section includes furnishing and installation of pull-out pivot wall mount systems designed for supporting flat panel audio-visual displays ranging from 43 inches to 55 inches in hospitality, residential, commercial, institutional, and professional environments.

Mounting system shall be Peerless-AV® Pull-Out Pivot Wall Mount, Model HPF650, or approved equal meeting all specified performance, swivel adjustment, pull-out access, compatibility, security, installation, and serviceability requirements without modification to the mounting system or accessories.

Includes wall mount assembly, universal adaptor brackets, mounting hardware, security hardware, cable management accessories, and manufacturer-approved accessories required for a complete installation.

Coordinate mounting locations with wall backing, recessed electrical boxes, low-voltage pathways, cable routing, and wall finishes to ensure proper installation, pull-out access, and viewing clearances.

RELATED SECTIONS

Division 06 - Wood: Blocking and backing within wall assemblies.

Division 09 - Finishes: Wall finishes, surface preparation, and non-load-bearing metal framing at mounting locations.

Division 26 - Electrical: Coordination of power supply, conduit, junction boxes, recessed outlet boxes, and data connections associated with wall-mounted displays.

Division 27 - Communications: Low-voltage cabling and cable routing associated with display installation.

REFERENCES

UL 2442 - Flat Panel Display Mounting Devices.

ADA Standards for Accessible Design, where applicable.

NFPA 70 - National Electrical Code (NEC).

ASTM C-90 - Standard Specification for Load-Bearing Concrete Masonry Units (applicable where concrete masonry unit (CMU) substrates are used).

AISI S100 - North American Specification for the Design of Cold-Formed Steel Structural Members.

AISI S211 - North American Standard for Cold-Formed Steel Framing.

Manufacturer’s published installation instructions and standard installation procedures.

Applicable local building codes and regulations governing mounting systems and accessibility.

SUBMITTALS

Product Data Sheets: Manufacturer’s current technical literature indicating dimensions, materials, finishes, load ratings, swivel ranges, pull-out extension distances, mounting patterns, and compliance listings.

Shop Drawings: Indicating mounting locations, pull-out clearances, viewing angles, and coordination with electrical and low-voltage systems.

Installation Instructions: Manufacturer’s current installation instructions and recommended anchorage methods.

Warranty Documentation: Manufacturer’s standard warranty information.

Accessory Data: Technical data for optional or specified accessories, if applicable.

QUALITY ASSURANCE

Manufacturer shall have a minimum of ten (10) years experience producing commercial A-V mounting systems of similar type, size, and complexity.

Products shall be UL listed and labeled by a nationally recognized testing laboratory and shall comply with applicable safety and performance standards.

Installer shall be experienced in installation of commercial AV mounting systems of similar size, weight and articulation complexity.

DELIVERY, STORAGE, AND HANDLING

Deliver products in original factory packaging with labels intact.

Store products indoors in a clean, dry location, protected from moisture, physical damage, and exposure to construction activities.

Protect from damage due to weather, moisture, construction operations and excessive temperature

This product is intended for indoor use only. Use outdoors may lead to product failure or personal injury.

WARRANTY

Provide manufacturer’s limited 5-year warranty covering defects in materials and workmanship under normal use and service conditions.

PRODUCTS

MANUFACTURERS

Basis-of-Design Product:
Peerless-AV® Pull-Out Pivot Wall Mount
Model: HPF650
Display Compatibility: 43” to 55” TVs
Manufactured by Peerless-AV®, 2300 White Oak Circle, Aurora, IL 60502; Phone (800) 865-2112, (630) 375-5100
Website: www.peerless-av.com

Substitutions:
Approved equal products shall meet or exceed all specified VESA compatibility, load capacities, pull-out extension capabilities, swivel ranges, auto-centering feature, in-furniture installation capability, mounting compatibility, installation methods, maintenance accessibility, and UL listing requirements without requiring redesign of wall conditions, structural reinforcement, or modification to accessories or mounting components, in accordance with Section 01 25 00.

PERFORMANCE REQUIREMENTS

Listings:

UL Listed and labeled in compliance with UL 2442 - Flat Panel Display Mounting Devices.

Comply with applicable environmental and material compliance requirements, including RoHS and TAA compliance.

Acceptable Wall Substrates:

Wood studs: Minimum 2” x 4” nominal size. Designed for 16” on center or single stud.

Steel studs: 16 inches on center or single stud configuration.

Load Capacity:
Mounting system shall support displays weighing up to 60 lb (27 kg) without permanent deformation or loss of structural integrity.

Display Size Range:
Compatible with flat panel displays ranging from 43 inches to 55 inches or larger, subject to display weight and VESA mounting pattern compatibility.

Mounting Pattern Compatibility:

Compatible with VESA® mounting patterns ranging from 200 x 200 mm to 400 x 400 mm.

I-shaped adaptor plate design avoids blocking connections on back of display.

Pull-Out and Swivel Functionality

Mounting system shall provide pull-out pivot functionality allowing maintenance and service access behind display assemblies.

Mounting system shall provide built-in swivel adjustment capability up to ±30 degrees for flexible viewing angle positioning.

Mounting system shall maintain centered display positioning during extension and retraction movement.

Fully retracted mount depth shall be approximately 2.32 inches (59 mm).

Fully extended mount depth shall be approximately 11.29 inches (287 mm).

Installation Features:

Mounting system shall incorporate pull-out pivot arm design providing simplified maintenance access behind display assemblies.

Provide adjustable latching mechanism allowing installer adjustment of closure tension when retracting display assembly against wall surface.

Provide security hardware to deter tampering and unauthorized removal of mounted display assemblies.

I-shaped adaptor plate configuration shall minimize obstruction of display power, signal, and connection locations.

Mounting system shall support recessed and in-furniture installation applications when utilized with manufacturer-approved accessories.

Design shall provide smooth extension and retraction movement while maintaining structural stability throughout swivel range.

PRODUCT SPECIFICATIONS

Physical Dimensions:

Overall Dimensions (W x H x D): 20.65” x 16.74” x 2.32”-11.29” (525 x 425 x 59-287 mm)

Product Weight: 13.40 lb (6 kg)

Load Capacity: 60 lb (27 kg)

Swivel Range: ±30° built-in horizontal swivel

Min VESA Pattern: 200 x 200 mm

Max VESA Pattern: 400 x 400 mm (15.75” x 15.75”)

Finish: Powder Coat, Black

Warranty: Limited 5-Year Warranty

Package Specifications:

Package Dimensions (W x H x D): 24.50” x 9.75” x 4.25” (622 x 248 x 108 mm)

Package Ship Weight: 14.81 lb (6.72 kg)

Package UPC: 735029298052

Package Contents: Wall mount, Universal Adaptor, wall and display installation hardware, installation instructions

Included Hardware:

Includes manufacturer-provided wall and display mounting hardware required for complete installation.

MATERIALS AND FINISH

Structural components shall be fabricated from heavy-duty steel suitable for supporting specified loads.

Finish shall be manufacturer’s standard powder coat finish.

Color: Black.

ACCESSORIES

Provide manufacturer-approved accessories compatible with the mounting system and installation requirements.

Accessories shall not reduce rated load capacity or interfere with mounting system performance.

Available accessories include, but are not limited to:

ACC-HPF650 - In-Furniture Bracket

ACC-UCM - Universal AV Component Mount

IBA3AC - Recessed Cable Management and Power Storage Accessory Box

EXECUTION

EXAMINATION

Verify wall construction, substrate type, and load-bearing capacity are suitable for installation of the pull-out pivot wall mount system and supported display loads.

Verify mounting locations, pull-out clearances, swivel movement range, and coordination with electrical and low-voltage systems prior to installation.

For wood stud walls: use a stud finder to locate and mark stud center lines. Confirm studs are a minimum of 2” x 4” nominal size and that gypsum board does not exceed 5/8” in thickness.

For metal stud walls: verify metal stud framing complies with AISI S100 or AISI S211 requirements and minimum gypsum board thickness requirements specified by manufacturer.

For concrete walls: confirm wall is at least 8” thick with a minimum compressive strength of 2,000 psi.

For concrete masonry unit (CMU) walls: confirm blocks meet ASTM C-90 specifications and have a minimum nominal width of 8”. Identify mortar joint locations to be avoided.

For in-furniture applications: confirm furniture opening dimensions, depth, and structural backing are compatible with flush HPF650 installation and ACC-HPF650 in-furniture bracket. Coordinate with furniture fabricator prior to installation.

INSTALLATION

Install mounting system in accordance with manufacturer’s written instructions and applicable codes.

Secure mounting system to structural framing or approved substrates using manufacturer-recommended fasteners and anchorage methods.

Coordinate mounting locations with wall blocking, electrical rough-in, recessed outlet boxes, and cable routing systems.

Verify mounting location provides adequate structural support for articulated loads generated during pull-out and swivel operation.

Verify adequate clearance for full extension and swivel operation prior to final installation.

Adjust mounting system for final level, alignment, extension range, and viewing orientation after installation.

Verify completed installation is secure, properly aligned, and fully operational prior to final acceptance.

CLEANING AND PROTECTION

Remove debris, packaging materials, and installation residue from project site.

Verify pull-out pivot and swivel operate freely without obstruction after cleanup.

Protect installed equipment from damage until substantial completion and final acceptance.

- END OF SECTION